Bayesian Optimization is a power JMP platform allowing the user to rapidly optimize one or several responses based on the supplied factors. I used BO to find the optimal PID values for a PID controller, using the values as my factors and the settling time and overshoot as my response. A JSL script handles supplying the factors and running the BO platform. I automated the process by closing the loop of BO, by having python mediate between my Aurdino based controller and the JSL running the BO platform.

The loop begins with some randomly generated seed values for the PID controller. These values are expected to not preform well. The seeds are stored in JSL. The JSL sends the first set of seeds to Python, which then sends them to the Aurdino over serial. When the Aurdiino receives the values, it tests them. The test consists of two main phases, the setup phase and return phase. The setup phase moves the motor away from the home position by a set amount. The return phase then brings the motor back to home, using the PID controller. Two responces are recored from each test, the overshoot in degrees and the settling time in seconds. The overshoot is how far over the home distance the motor travels, and the settling time is how long the motor takes to stop at the home position. A properly tunes PID should have no overshoot and should reach home as quickly as possible, so we want to minizie both of these responces.